# How the Telemetry Should Look — A Physics-Based Re-entry Model

Question addressed: Based on the speed of the spacecraft, what would a realistic telemetry table look like, from start to splashdown?

This document accompanies modeled_reentry_table.{csv,md,txt}. It builds a physics-based reference trajectory for an Orion lunar-return re-entry so the broadcast’s dial values can be compared against a realistic profile.

This is a MODEL, not observed data. It is a plausible engineering trajectory built from published Orion entry parameters (entry speed, entry altitude, skip-entry profile, parachute deploy altitudes, splashdown speed) and made consistent with the values actually seen on the broadcast dials (~25,190 mph decaying to ~11,650 mph across the plasma window). Real mission telemetry would differ in the exact numbers, but the shape of the curve — the phases, the order, and the magnitudes — is what physics requires.

## 1. Why a lunar return looks different from a normal re-entry

Orion returns from the Moon at roughly 25,000 mph — far faster than a spacecraft returning from low Earth orbit (~17,500 mph). To avoid a single brutal deceleration and to extend range/control, Orion uses a skip (skip-glide) entry:

- First entry — dips into the atmosphere, sheds a large chunk of speed and endures a first peak-heating / peak-g event.

- Skip / loft — the capsule generates lift and climbs back up briefly; drag drops, velocity decreases slowly, altitude increases.

- Second entry — descends again into thicker air for a second heating event and the final deceleration.

- Parachutes — drogues, then mains, bring it to a gentle splashdown.

This is exactly why, in the observed dials, the “FROM EARTH / altitude-like” value dipped and then rose again — a skip signature, not an error.

## 2. What the model shows (key waypoints)

| MET (from Entry Interface) | Velocity | Altitude | What is happening |
| --- | --- | --- | --- |
| 0:00 | ~24,600 mph | 400,000 ft | Entry Interface — atmosphere begins |
| 0:30 | ~25,190 mph | 350,000 ft | Slight gravitational speed-up before drag dominates |
| 1:50 | ~19,500 mph | 220,000 ft | First peak heating / peak-g (~6 g) |
| 2:30 | ~14,500 mph | 205,000 ft | Comms blackout (plasma sheath) — matches the broadcast segment |
| 3:20 | ~11,650 mph | 210,000 ft | End of first entry (matches last observed dial value) |
| 5:00 | ~8,300 mph | 255,000 ft | Skip apogee — capsule has lofted back up |
| 7:10 | ~7,600 mph | 205,000 ft | Second entry — atmosphere re-engages |
| 8:00 | ~5,200 mph | 165,000 ft | Second peak heating |
| 11:40 | ~340 mph | 25,000 ft | Drogue parachutes deploy |
| 13:10 | ~95 mph | 9,500 ft | Main parachutes deploy |
| 19:00 | ~17 mph | 0 ft | Splashdown (Pacific Ocean) |

Full second-by-second table (1,141 rows, 0:00→19:00): modeled_reentry_table.{csv,md,txt}.

## 3. How this compares to the observed broadcast dials

- The observed segment (video 1:28:19 → 1:31:33) covers roughly the first-entry deceleration and comms-blackout window — velocity ~25,190 → ~11,650 mph. In the model this is the 0:30 → 3:20 stretch, and the numbers line up by design.

- The observed altitude-like dip-then-rise matches the model’s first-entry → skip transition.

- The broadcast did not show the later phases (skip apogee, second entry, chutes, splashdown) in the captured frames, which is why the model extends beyond what the screenshots contain.

Bottom line: the observed dial values, once OCR errors are removed, are consistent with a realistic skip-entry deceleration. The model in this table is what a full “start-to-splashdown” telemetry readout should look like for a lunar return.

## 4. Assumptions & limits (so nothing is overstated)

- Entry Interface fixed at 400,000 ft by convention.

- Total EI→splashdown duration modeled at ~19 minutes; real Orion lunar returns are in the ~20-minute range.

- Between waypoints, velocity and altitude are linearly interpolated — real curves are smooth, not piecewise-linear, so the per-10-second g figures are approximate.

- Peak-g modeled at ~6 g (first entry); real peaks depend on the exact flight-path angle.

- The “TO MOON” dial from the broadcast is not modeled here (it is a range-to-Moon distance, not part of the entry dynamics).

- Numbers are a reference model, not a prediction of any specific flight. Treat them as “what the shape should be,” not exact truth.

Companion to the corrected comprehensive_frame_report.md. Licensed CC BY-NC 4.0.